dgSCOM Dongle Assembly

The dgSCOM dongle has 3 pieces. The I2C interface board (PCB), the USB 2.0 to Serial FIFO Bridge Interface Board (UM232H), and a two-pin jumper.

Assemble the dongle by first plugging UM232H on to the U1 sockets (see left picture), then plugging in the two-pin jumper on J1. When the dongle is assembled correctly and D2XX driver is installed on a PC, the LED1 PWR should light up when plugging into a PC via mini-USB.

D2XX driver: https://ftdichip.com/drivers/d2xx-drivers/

I2C Interface Module and the dgSCOM software/GUI Guide

I2C Interface Module

Install dgSCOM software/GUI with dgSCOM Setup.exe.

After installing dgSCOM successfully, dgSCOM.exe ICON will appear on the desktop. Click on dgSCOM.exe to lunch the GUI.

Using dgSCOM software/GUI

dgSCOM should recognize the I2C Interface Module as UM232H. Click the Connect button to establish connection between the GUI and the I2C Interface Module. FT_OK shown after connecting the GUI to the I2C Interface Module properly.